# Year 11 Electricity and Magnetism Practice Paper

Exam style questions   •   60 marks   •   Suggested time 70 minutes

Name ____________________________________    Class __________    Date ______________

Answer every question. Show your working and include units in calculations. Write or draw in the spaces provided. Use the circuit figures for all numerical questions; the photographs provide context and do not specify the component values.

Useful equations: V = I × R    I = V ÷ R    R = V ÷ I    P = V × I    B = KI ÷ d
Total series resistance = R₁ + R₂    K = 2.0 × 10⁻⁷ T m/A

# Question 1  Static electricity                                      [8 marks]

A pupil rubs a plastic rod with a wool cloth. The rod becomes negatively charged and attracts small neutral paper pieces.

(a)  Describe the transfer of electrons between the cloth and rod, and state the charge left on the cloth.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(b)  Explain why the charged rod attracts a neutral paper piece.   [3]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(c)  Explain how a charged cloud can cause a lightning discharge to the ground.   [3]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

## Photograph sources and licences

Meganbeckett27: https://commons.wikimedia.org/wiki/File:Electric_circuit_with_voltmeter_and_ammeter.JPG  •  https://creativecommons.org/licenses/by-sa/3.0/  (cropped)

Z22: https://commons.wikimedia.org/wiki/File:Exit_sign_-_inside.jpg  •  https://creativecommons.org/licenses/by-sa/4.0/  (cropped)

SeanShriner: https://commons.wikimedia.org/wiki/File:Nail_Electromagnet.JPG  •  https://creativecommons.org/licenses/by-sa/3.0/  (cropped)

# Question 2  Meters and a lamp                                      [10 marks]

A student tests a lamp for a lighting project. The photograph shows an example of laboratory meter apparatus. The circuit figure below shows the student’s test circuit; meters X and Y are unidentified. Assume the full 9 V is across the lamp.

Photograph by Meganbeckett27, Wikimedia Commons, CC BY-SA 3.0; cropped. Source and licence links on page 1.

(a)  Identify meters X and Y. Explain why each is connected in its shown position.   [4]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(b)  X reads 0.30 A. Calculate the resistance of lamp L at this operating point. Show your equation, substitution and answer with a unit.   [3]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(c)  Calculate the power transferred by the lamp.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(d)  State the unit used for electrical power.   [1]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

# Question 3  Lamps in series                                      [10 marks]

The photograph shows two lamps in an opened emergency light. A student uses the circuit figure below to model two lamps A and B connected in series to a 12 V supply. For this question their resistances are 4 Ω and 8 Ω; treat them as constant.

Photograph by Z22, Wikimedia Commons, CC BY-SA 4.0; cropped. Source and licence links on page 1.

(a)  Calculate the total circuit resistance.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(b)  Calculate the current through lamp A.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(c)  Calculate the potential difference across lamp B.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(d)  State the potential difference across lamp A.   [1]

_____________________________________________________________________________________________

(e)  Another lamp is added in series. Explain what happens to the total resistance, current and brightness of the original lamps.   [3]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

# Question 4  Lamps in parallel                                      [10 marks]

A student models the lighting in two rooms using the circuit figure below. Each room has one lamp; only the lamp in room B has switch S. S is closed at first. The lamp resistances are constant. Assume an ideal 6 V supply.

(a)  State the potential difference across each lamp.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(b)  Calculate the current through lamp A.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(c)  Calculate the current through lamp B.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(d)  Calculate the total current supplied by the battery.   [1]

_____________________________________________________________________________________________

(e)  Calculate the power transferred by lamp B.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(f)  Switch S is opened. State the new current supplied by the battery.   [1]

_____________________________________________________________________________________________

# Question 5  Magnetic field calculations                                      [10 marks]

A long straight wire carries a current. The magnetic field strength B at a distance d from its centre is B = KI ÷ d. Use K = 2.0 × 10⁻⁷ T m/A and distances in metres.

(a)  The current is 4.0 A. Calculate B at 5.0 cm from the wire. Show the distance conversion and your working.   [3]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(b)  At d = 0.020 m, B = 3.0 × 10⁻⁵ T. Calculate the current using I = Bd ÷ K.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(c)  A wire carries 5.0 A. Calculate the distance at which B = 2.0 × 10⁻⁵ T. Use d = KI ÷ B.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(d)  State what happens to B if only the current doubles. Then state what happens to B if only the distance doubles.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(e)  State the unit used for magnetic field strength B.   [1]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

# Question 6  Magnetic fields and a door lock                                      [12 marks]

The photograph shows wire wound around an iron core to make an electromagnet. A school model uses a similar coil with a soft iron core to attract an iron armature connected to a spring-loaded door latch. The figure below shows a separate bar magnet near an unmagnetised iron nail.

Photograph by SeanShriner, Wikimedia Commons, CC BY-SA 3.0; cropped. Source and licence links on page 1.

(a)  State the pole induced at the end of the nail nearest the magnet and explain why the nail is attracted.   [2]

_____________________________________________________________________________________________

(b)  On the diagram, draw two external magnetic field lines around the bar magnet, including their direction arrows.   [2]

(c)  Describe the shape of the magnetic field lines around a straight wire carrying a current.   [1]  Answer: __________________________

(d)  State one feature of the field lines inside a long solenoid that shows the field is approximately uniform.   [1]  Answer: __________________________

(e)  Explain how closing and opening a switch moves the armature and operates the latch. Include the role of a soft iron core and two changes that would make the electromagnet stronger.   [6]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________