# Year 11 Electricity and Magnetism Practice Paper

Original GCSE style questions   •   60 marks   •   Suggested time 70 minutes

Name ____________________________________    Class __________    Date ______________

Answer every question. Show your working and include units in calculations. Write or draw in the spaces provided. Use the circuit figures for all numerical questions; the photographs provide context and do not specify the component values.

Useful equations:   V = IR      P = VI      total series resistance = R₁ + R₂ + ...

# Question 1  Static electricity                                      [8 marks]

A pupil rubs a plastic rod with a wool cloth. The rod becomes negatively charged and attracts small neutral paper pieces.

(a)  Describe the transfer of electrons between the cloth and rod, and state the charge left on the cloth.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(b)  Explain why the charged rod attracts a neutral paper piece.   [3]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(c)  Explain how a charged cloud can cause a lightning discharge to the ground.   [3]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

## Photograph sources and licences

Meganbeckett27: https://commons.wikimedia.org/wiki/File:Electric_circuit_with_voltmeter_and_ammeter.JPG  •  https://creativecommons.org/licenses/by-sa/3.0/  (cropped)

Z22: https://commons.wikimedia.org/wiki/File:Exit_sign_-_inside.jpg  •  https://creativecommons.org/licenses/by-sa/4.0/  (cropped)

SeanShriner: https://commons.wikimedia.org/wiki/File:Nail_Electromagnet.JPG  •  https://creativecommons.org/licenses/by-sa/3.0/  (cropped)

# Question 2  Meters and a lamp                                      [10 marks]

A student tests a lamp for a lighting project. The photograph shows an example of laboratory meter apparatus. The circuit figure below shows the student’s test circuit; meters X and Y are unidentified. Assume the full 9 V is across the lamp.

Photograph by Meganbeckett27, Wikimedia Commons, CC BY-SA 3.0; cropped. Source and licence links on page 1.

(a)  Identify meters X and Y. Explain why each is connected in its shown position.   [4]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(b)  X reads 0.30 A. Calculate the resistance of lamp L at this operating point. Show your equation, substitution and answer with a unit.   [3]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(c)  Calculate the power transferred by the lamp.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(d)  State what a potential difference of 9 V means for each coulomb of charge passing through the lamp.   [1]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

# Question 3  Lamps in series                                      [10 marks]

The photograph shows two lamps in an opened emergency light. A student uses the circuit figure below to model two lamps A and B connected in series to a 12 V supply. For this question their resistances are 4 Ω and 8 Ω; treat them as constant.

Photograph by Z22, Wikimedia Commons, CC BY-SA 4.0; cropped. Source and licence links on page 1.

(a)  Calculate the total circuit resistance.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(b)  Calculate the current through lamp A.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(c)  Calculate the potential difference across lamp B.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(d)  State the potential difference across lamp A.   [1]

_____________________________________________________________________________________________

(e)  Another lamp is added in series. Explain what happens to the total resistance, current and brightness of the original lamps.   [3]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

# Question 4  Lamps in parallel                                      [10 marks]

A student models the lighting in two rooms using the circuit figure below. Each room has one lamp; only the lamp in room B has switch S. S is closed at first. The lamp resistances are constant. Assume an ideal 6 V supply.

(a)  State the potential difference across each lamp.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(b)  Calculate the current through lamp A.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(c)  Calculate the current through lamp B.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(d)  Calculate the total current supplied by the battery.   [1]

_____________________________________________________________________________________________

(e)  Calculate the power transferred by lamp B.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(f)  Switch S is opened. State the new current supplied by the battery.   [1]

_____________________________________________________________________________________________

# Question 5  A mixed circuit                                      [10 marks]

A student models a low-voltage greenhouse heater using two 8 Ω heating elements. The circuit figure shows a 2 Ω resistor in series with the two parallel elements. The supply is 12 V and the ammeter reads 2.0 A. Assume ideal components.

(a)  Calculate the potential difference across the 2 Ω resistor.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(b)  Calculate the potential difference across the parallel branches.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(c)  Calculate the current in each 8 Ω branch.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(d)  Show that your answers obey both the junction rule for current and the loop rule for potential difference.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

(e)  One 8 Ω branch breaks. Calculate the new total circuit resistance and battery current.   [2]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

# Question 6  Magnetic fields and a door lock                                      [12 marks]

The photograph shows wire wound around an iron core to make an electromagnet. A school model uses a similar coil with a soft iron core to attract an iron armature connected to a spring-loaded door latch. The field-line figure below shows a separate bar magnet near an unmagnetised nail and a wire viewed end-on; the dot means conventional current comes out of the page.

Photograph by SeanShriner, Wikimedia Commons, CC BY-SA 3.0; cropped. Source and licence links on page 1.

(a)  State the pole induced at the end of the nail nearest the magnet and explain why the nail is attracted.   [2]

_____________________________________________________________________________________________

(b)  On the diagram, draw two external magnetic field lines around the bar magnet, including their direction arrows.   [2]

(c)  On the diagram, draw one circular magnetic field line around the wire, with a direction arrow.   [1]

(d)  Viewed from one end of a solenoid, current flows anticlockwise. State which magnetic pole this end becomes.  Answer: __________   [1]

(e)  Explain how closing and opening a switch moves the armature and operates the latch. Include the role of a soft iron core and two changes that would make the electromagnet stronger.   [6]

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________

_____________________________________________________________________________________________